Task-1 Create a comparison table listing the main differences between DDR3, DDR4, and DDR5 RAM, including speed, voltage, and typical use cases.

| Feature / Parameter | DDR3 | DDR4 | DDR5 |
| --- | --- | --- | --- |
| Release Era | 2007 | 2014 | 2020 |
| Transfer Speed (MT/s) | 800 – 2133 MT/s | 1600 – 3200+ MT/s | 4800 – 8400+ MT/s |
| Operating Voltage | 1.5V (Standard) / 1.35V (Low Voltage) | 1.2V | 1.1V (Base) |
| Max Capacity per Module | Typically up to 16 GB | Typically up to 64 GB | Up to 128 GB+ |
| Power Management | Managed by motherboard | Managed by motherboard | On-DIMM PMIC (Power Management IC) |
| Typical Use Cases | Legacy systems, older office desktops, budget builds | Mainstream gaming PCs, mid-range laptops, older servers | High-end gaming rigs, heavy content creation workstations, modern servers |

Task-2 Open your laptop or desktop (with power disconnected), locate the RAM slots, and take a clear photo of the installed RAM module. Reinstall the RAM module and verify that your system boots up successfully.Hint:Refer to your device manual or a trusted YouTube guide if unsure about RAM removal and installation steps.

Task-3 List three key differences between HDD, SSD, and NVMe storage types, and for each, name one app or use case (from apps you use daily) that would benefit most from that storage type.

### Differences

| Feature / Parameter | HDD (Hard Disk Drive) | SATA SSD (Solid State Drive) | NVMe SSD (Non-Volatile Memory Express) |
| --- | --- | --- | --- |
| Underlying Technology | Mechanical spinning magnetic platters and a moving read/write head. | Flash memory (NAND) communicating via the legacy SATA bus interface. | Flash memory (NAND) communicating directly over the high-speed PCIe bus. |
| Read / Write Speeds | Slow (~80 to 160 MB/s) due to mechanical bottlenecks. | Moderate (~500 to 550 MB/s), maxing out the limits of the SATA III interface. | Blazing Fast (3,500 to 14,000+ MB/s) depending on the PCIe generation (Gen 3, Gen 4, or Gen 5). |
| Cost & Capacity Efficiency | Lowest cost per gigabyte; ideal for massive, bulk storage capacities. | Moderate cost per gigabyte; a solid standard upgrade for older systems. | Higher cost per gigabyte for cutting-edge performance and dense data throughput. |

### Storage Types and Ideal Daily Apps

- HDD (Mass Archival & Backup)

  - Best Use Case / App: Plex Media Server / Long-Term File Backups — Excellent for storing terabytes of movies, music archives, system backups, and raw project files where lightning-fast access speeds aren't critical, but high storage capacity at a low cost is essential.

- SATA SSD (Standard Productivity)

  - Best Use Case / App: Google Chrome / General Web Browsing — Ideal for everyday multitasking, office document editing, and web browsing, providing a noticeable speed boost over mechanical drives for launching standard desktop applications.

- NVMe SSD (High-Performance & Development)

  - Best Use Case / App: Visual Studio Code / VMware Workstation (Virtual Machines) — Perfect for software development environments, compiling large codebases, managing heavy node_modules folders, and running virtual machines (like Kali Linux) where instantaneous file access and heavy read/write input-output operations prevent system bottlenecks.

Task-4 Remove the storage drive (HDD or SSD) from your system, take a photo of the connector type (SATA, M.2, or NVMe), and reinstall the drive. Confirm that your device recognizes the storage after reassembly.Constraint: Only perform this task if you have access to a device you are allowed to open. If not, find and share a YouTube video link showing the process for your laptop/PC model.